# Project Title: Graphical Convolution GUI in MATLAB

(CHATGPT AIDED AND EDITED)

## Objective

Build a MATLAB GUI (extending Dr. Uysal’s base file) that:
1) Performs General Graphical Convolution between user-defined signals, and
2) Provides a Probability & Statistics Mode where signals are treated as PDFs and statistical properties are analyzed.
Users choose the mode on launch via a two-button start screen.

## GUI Flow (Two Choices at Launch)

Startup Screen (Mode Selector):
- Button 1: General Graphical Convolution
- Button 2: Probability & Statistics Mode

Selecting a mode transitions to the corresponding workspace (single window with stacked panels or tabbed UI). Include a 'Back to Mode Selector' action to switch modes without restarting.

## Requirements

### 1) Use of Base Code

Must start from and integrate Dr. Uysal’s provided file. You may add functions, components, and structure, but the base logic should remain identifiable.

### 2) Common (Shared) UI Elements

Signal Definition for x(t), h(t): Templates (rect, tri, exp, sinusoid, step, ramp) or custom expressions.
Adjustable parameters: amplitude, duration, start/end times, time step (fractional allowed).

Controls: Generate Signals, Run Convolution, Pause/Resume, Reset, and Back to Mode Selector.

Displays:
- Original signals
- Flipped/shifted h(t)
- Pointwise product x(τ)*h(t−τ)
- Accumulated integral (summation)
- Final output y(t)=x*h

Property Visuals (both modes): Width property and Shift property should be clearly visualized.

### 3) Mode-Specific Behavior

A. General Graphical Convolution (Mode 1)

Manual convolution visualization (no conv() for the main process). Smooth animation or single-step playback. Optional “Verification” overlay: compute conv() separately and plot for comparison.

B. Probability & Statistics Mode (Mode 2)

Treat x(t), h(t) as PDFs: enforce non-negativity and normalize to 1 (provide a 'Normalize' toggle).
Compute and display:
- Means (μx, μh, μy) and variances (σx², σh², σy²).
- Check: μy = μx + μh, σy² = σx² + σh² (independent random variables).
- Normalization checks: ∫x(t)dt = ∫h(t)dt = ∫y(t)dt = 1.
Optional Monte Carlo tab (bonus): simulate sums of two independent RVs and overlay histogram vs. analytical convolution.

### 4) Data & Validation

Support non-integer time steps (e.g., 0.1, 0.25, 0.5).
Provide user feedback for invalid expressions or unnormalized PDFs.
Allow exporting plots and results for documentation.

## Deliverables

1) MATLAB GUI Application using Dr. Uysal’s base file, with two-mode startup and all visuals.
2) Documentation (Report): Describe GUI design, base code integration, width/shift demonstrations, and probability mode.
3) Presentation: PPTX + YouTube demo link demonstrating both modes.
4) Oral Exam: Discussion on theory, implementation, and design.

## Evaluation Criteria

| Category | Description |
| --- | --- |
| Use of Base Code | Properly extends and integrates Dr. Uysal’s file. |
| Two-Mode UX | Clear startup selector; smooth switching; coherent layouts. |
| Functionality | Correct manual convolution; accurate width & shift visuals; correct probability/statistics computations. |
| Visualization Quality | Stepwise overlap/integral clear; annotations informative. |
| Usability | Intuitive inputs, validation messages, and controls. |
| Code Quality | Modular, documented, and robust. |
| Docs & Presentation | Complete, clear, and well-illustrated; includes both modes. |